Security Issues in IoT, PLC, and Embedded Devices

CYBR3020

Vulnerabilities and Exploits

Arr Domingo

Student ID: 200458099

December 8, 2025

Instructor: Clayton Amelia

Table of Contents

1	Introduction	1

2	Cybersecurity Issues, Exploit Process, and Mitigation	1

2.1	Issue 1: Legacy Industrial Protocols	1

2.1.1	About the issue	1

2.1.2	How it can be exploited	2

2.1.3	Remediation / Mitigation	2

2.2	Issue 2: Default Credentials and Weak Access Controls	2

2.3	Issue 3: Lack of Firmware Updates and Patch Management	2

2.4	Issue 4: Unmanaged Logic & Configuration Lifecycle	3

2.5	Issue 5: Physical Access Vulnerabilities	3

3	Incident - PLC Compromises in US Water Utilities	3

3.1	About the real-world example	3

3.2	Real world example prevention strategy	4

References	1

# Introduction

This document will focus on the security issues of PLC (Programmable Logic Controllers). PLCs are industrial computers, with various inputs and outputs, used to control and monitor industrial equipment based on custom programming such as manufacturing, water treatment, energy, transportation, and chemical production. They serve as the backbone of industrial operations because PLCs interface directly with physical equipment like sensors, pumps, motors, valves and robotic arms. Historically, PLCs were designed for reliability and safety, not for modern network-connected environments. With the integration of industrial networks and insufficient built-in security controls, this made PLC a prime target to cybersecurity threats and cyberattacks. Therefore, this document analyzes key security issues affecting PLCs, explains how attackers exploit these vulnerabilities, and outlines mitigation strategies. A real-world case study involving the water treatment?? demonstrates how PLC security weaknesses can result in major physical and operational consequences.

# 2	Cybersecurity Issues, Exploit Process, and Mitigation

In today’s operational technology (OT) threat landscape, PLC become an attack surface for an easy entry point to industrial facilities. Below are issues related to PLC.

## 2.1	Issue 1: Legacy Industrial Protocols

### 2.1.1	About the issue

Many ICS (Industrial Control System) protocols lack built‑in authentication, encryption, message integrity, or robust replay protections. These protocols were originally designed for closed, isolated networks and lack essential security features like authentication, integrity checking, and encryption. When these protocols traverse IP networks, attackers can inject, replay, or tamper with control messages to read/write registers and manipulate device states.

### 2.1.2	How it can be exploited

Attackers can perform command injections by sending unauthorized write commands to change device logic. Without encryption, threat actors can sniff traffic, capture process data, or manipulate values in transit. Also, man-in-the-middle attacks allow adversaries to alter PLC instructions without detection by operators.

### 2.1.3	Remediation / Mitigation

It is advisable to use VLANs, firewalls, and segmentation to limit access to the control network. Moreover, it is also helpful to enable encrypted tunnels such as VPNs for remote access and monitor network anomalies using ICS-aware intrusion detection systems.

## 2.2	Issue 2: Default Credentials and Weak Access Controls

### 2.2.1	About the issue

PLCs and the engineering workstations still rely on default vendor usernames/passwords like admin/admin or password123 because it was not originally intended to be internet-accessible, that is why security was minimal. Additionally, many industrial devices lack support for modern authentication methods which frequently fall to credential abuse, enabling configuration access, program uploads, or unsafe command execution.

### 2.2.2	How it can be exploited

Attackers performing reconnaissance find exposed devices with open ports (e.g., port 502 for Modbus). Then use default passwords to access engineering workstations, upload altered logic or stop processes. The hardcoded credentials allow attackers to fully compromise a PLC without needing user interaction.

### 2.2.3	Remediation / Mitigation

Enforce mandatory password changes, role-based access control and disable unused accounts and services. Use multi-factor authentication (MFA) for engineering workstations and utilize vendor tools that allow password rotation and credential auditing.

## Issue 3: Lack of Firmware Updates and Patch Management

### 2.3.1	About the issue

PLCs often remain in operation for decades, which is beyond their expected support life. Vendors may only provide patches for critical issues, and many organizations avoid updating firmware due to fear of disrupting operations, limited maintenance windows, compatibility issues with old hardware, lack of vendor patches, and difficulty identifying firmware versions. This creates large attack windows where outdated devices remain vulnerable.

### 2.3.2	How it can be exploited

Attackers reverse-engineer outdated firmware to find vulnerabilities such as buffer overflows or authentication bypasses. The publicly known vulnerabilities in older PLC models can be exploited remotely if patches were never applied.

### 2.3.3	Remediation / Mitigation

Maintain an asset inventory with firmware versions and vendor support timelines. Schedule controlled maintenance windows for applying updates safely. Isolate outdated equipment on separate network segments. Implement compensating controls such as intrusion monitoring, strict firewall rules, and physical access restrictions.

## Issue 4: Unmanaged Logic & Configuration Lifecycle

### 2.4.1	About the issue

Poor program management like missing checksums/signing, weak version control, and ad‑hoc backups leads to tampering and drift. Attackers exploit unmanaged artifacts such as outdated project files on engineering laptops/servers to inject logic then deploy unnoticed where operators may not detect subtle changes until damage occurs.

### 2.4.2	How it can be exploited

Using stolen or discovered project files, adversaries can recompile with malicious routines or leverage vendor compare tools only after compromise is suspected. Industrial configuration lacked encryption or integrity verification.

Modify ladder logic/function blocks; install payloads that falsify readings or silently alter timings; in advanced cases, deploy PLC rootkits to mask changes from operators (as Stuxnet did via tampered Step7 DLLs and PLC code).

### 2.4.3	Remediation / Mitigation

Policy and Operational Measures

- Train technicians and engineers in secure configuration practices.

- Maintain up-to-date documentation of assets, logic versions, and firmware.

- Conduct periodic vulnerability assessments and penetration tests (within policy).

## Issue 5: Physical Access Vulnerabilities

### 2.5.1	About the issue

PLCs are often installed in easily accessible locations like factory walls, pump stations, utility boxes, and control cabinets. Because many PLCs include unprotected interfaces and exposed programming ports such as USB or serial ports, physical access can be enough to compromise them completely.

### 2.5.2	How it can be exploited

An attacker with brief access might easily configure PLC firmware, can reveal memory contents, and bypass authentication that can compromise engineering laptops or the PLC itself.

### 2.5.3	Remediation / Mitigation

Secure control cabinets with locks, monitoring, and restricted access. Disable or password-protect unused programming ports. Implement tamper-evident seals. Store engineering laptops in secure areas and enforce USB device control policies.

# Incident – Unitronics PLC in Water Utilities

## 3.1	About the real-world example/incident

In November 2023, the Municipal Water Authority of Aliquippa (Pennsylvania) reported that a Unitronics Vision PLC controlling a booster station had been compromised. The utility responded by taking the device offline and switching to manual operations, and regulators noted no risk to drinking water. Federal advisories connected the activity to IRGC‑affiliated CyberAv3ngers, stressing the role of default passwords and internet exposure in enabling access.

Technical/Procedural Notes.

- Attackers leveraged publicly exposed PLCs (default TCP 20256) and weak credentials to access HMIs and display defacements; ICS guidance recommended changing defaults, MFA, VPN, and allow-listing. [cisa.gov], [krwa.org]

- Federal and media reports indicated multiple U.S. facilities and even an aquarium were affected across states—showing sector-wide exposure when defaults and open connectivity persist.

## 3.2	Prevention strategy real world example/incident

Mitigation Lessons for Water Utilities.

- Eliminate defaults and audit for weak passwords across all PLCs/HMIs. 2) Remove direct exposure; place remote stations behind secure gateways with MFA. 3) Monitor protocol traffic and block/alert on Unitronics PCOM/TCP scans and unauthorized writes. 4) Practice manual-operation drills for booster/pump stations to ensure resilience.

# References

How Attackers Exploit ICS Protocols—And How to Stop Them. (2025, July 16). Retrieved from Insane Cyber: https://insanecyber.com/ics-protocol-security-modbus-dnp3-defense-guide/

Moukbel, T. (n.d.). The Hidden Dangers of Unmanaged PLC Programs: Cybersecurity Risks and Best Practices. Retrieved from Undercode Testing: https://undercodetesting.com/the-hidden-dangers-of-unmanaged-plc-programs-cybersecurity-risks-and-best-practices/

PLC: Programmable Logic Controller. (2025, October 9). Retrieved from Inductive Automation: https://inductiveautomation.com/resources/article/what-is-a-PLC

PLCs security risks. (2024, May 30). Retrieved from Incibe: https://www.incibe.es/en/incibe-cert/blog/plcs-security-risks